SECTION 07 72 53.01

SNOW GUARDS

This section has been prepared by Metal Roof Innovations, Ltd. for use in the preparation of a project specification section covering ColorGard snow guards with color-matched metal strip inserts. Attachment may be by one of the following methods:

S-5! Clamps: A non-penetrating system for use on standing seam metal roofs.

VersaBracket: A face-fastened (penetrating) system for use on exposed fastener metal panels, tiles and shingles and selected other roof types.

The following should be noted in using this specification:

Notes are included to assist the user in editing the section to suit project requirements. These notes are included as hidden text, and can be revealed or hidden by one of the following methods:

Microsoft Word: From the pull-down menus select TOOLS, then OPTIONS. Under the tab labeled VIEW, select or deselect the HIDDEN TEXT option.

Corel WordPerfect: From the pull-down menus select VIEW, then select or deselect the HIDDEN TEXT option.

Hypertext links to specific websites are included after manufacturer names and names of organizations whose standards are referenced within the text, to assist in product selection and further research. Hypertext links are contained in parenthesis, e.g.:

(www.S-5.com)

Optional text requiring a selection by the user is enclosed within brackets, e.g.: "Section [09 00 00.] [_____.]"

Items requiring user input are enclosed within brackets, e.g.: "Section [_____ - ________]."

Optional paragraphs are separated by an "OR" statement, e.g.:

**** OR ****

"Green" requirements are included for projects requiring LEED certification, and are included as green text. For additional information on LEEDS, visit the U.S. Green Building Council website at www.usgbc.org.

The following guide specifications on related products are also available from Metal Roof Innovations, Ltd.:

Section 07 72 53.02 - Snow Guards: Snow guards for standing seam metal roofs, using the S-5! non-penetrating attachment method.

Section 07 72 55 - Roof Accessory Attachment Systems: A non-penetrating system for attachment of equipment, piping, conduit, snow guards, walkways, parapet bracing, solar panels, and other accessories to standing seam metal roofs.

For assistance on the use of the products in this section, contact Metal Roof Innovations, Ltd. by calling 719-495-0518, by email at info@S-5solutions.com, or visit our website at www.S-5.com.

- - GENERAL

  - SUMMARY

- Edit the following paragraphs to include only those items specified in this section.

    - Section Includes:

      - Snow guards for metal roofs.

      - [Non-penetrating] [Face fastened] attachment system.

      - Color-matched metal strips.

- Coordinate the following paragraphs with other sections in the project manual.

    - Related Sections:

      - Division 1: Administrative, procedural, and temporary work requirements.

      - Section [07 41 00 - Metal Roof Panels:] [07 61 00 - Sheet Metal Roofing:] [_____ - _______:] Metal roof panels.

      - Section 07 72 55 - Roof Accessory Attachment System.

  - REFERENCES

    - Aluminum Association (AA) (www.aluminum.org) - Aluminum Standards and Data, 2003 Edition.

    - ASTM International (ASTM) (www.astm.org):

      - B85-03 - Standard Specification for Aluminum-Alloy Die Castings.

      - B221-04a - Standard Specification for Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes.

  - SYSTEM DESCRIPTION

- Include the following for snow guards installed using S-5! clamps on standing seam roofs only; delete for roofs using VersaBracket attachment.

    - Attachment system to provide attachment to standing seam metal roofs:

      - With only minor dimpling of panel seams.

      - Without penetrations through roof seams or panels.

      - Without use of sealers or adhesives.

      - Without voiding roof warranty.

- Roof snow loads that can be expected for any given project vary based on project location, roof dimensions, roof slope, wind effects, and building configuration. The following paragraph establishes the minimum in-service vector load in pounds per linear foot of eave, which is then used to design the attachment system. This vector load must be determined from the site-specific details by a structural engineer or in conjunction with the S-5! ColorGard distributor. Note that the in-service vector load is not the same as the code-dictated snow load based on horizontal at-grade surfaces.

    - Loading: Design snow guard system to resist minimum in-service vector load of [__] pounds per linear foot of eave.

    - Factor of safety:  Utilize a factor of safety  [2] [____] to determine allowable loads from ultimate tested clamp tensile load values.

  - SUBMITTALS

- Limiting submittals to only those actually required helps to minimize liability arising from the review of submittals. Minimize submittals on smaller, less complex products.

- Include the following for submission of shop drawings, product data, and samples for the Architect's review.

    - Submittals for Review:

      - Shop Drawings: Show locations of snow guards on roof and attachment spacing.

      - Product Data: Include product description and installation instructions.

      - Samples:

        - Clamp samples.

        - 24 inch long cross member samples including color-matched metal strip, splice connector, and other hardware.

    - Quality Control Submittals:

      - Test results: Results of product tensile load testing, issued by a recognized independent testing laboratory, showing ultimate load-to-failure value of attachment.

- Include the following for submission of sustainable design submittals.

    - Sustainable Design Submittals:

- Include the following for projects located within 500 miles of the Metal Roof Innovations, Ltd. fabrication facility.

      - Regionally manufactured products: Certify location of material manufacturer and distance from manufacturer to project site.

- Include the following for submission of closeout submittals for the Owner's record purposes.

    - Closeout Submittals:

      - Certification: Installer's certification that snow guard system was installed in accordance with manufacturer's instructions and approved Shop Drawings.

  - QUALITY ASSURANCE

- Include the following for full size mockups for review of construction and coordination of work of several sections.

    - Mockup:

      - Size: Minimum [8] [__] feet long.

      - Show: Snow guard attachment, cross members, and accessories.

      - Locate [where directed.] [____.]

      - Approved mockup may remain as part of the Work.

- - PRODUCTS

  - MANUFACTURERS

    - Contract Documents are based on S-5! ColorGard as represented & distributed locally by :

Engineered Architecturals Ltd

1-800-737-5811

info@engineeredarchitecturals.com

www.engineeredarchtiecturals.com (www.S-5.com)

- Edit the following to indicate whether or not substitutions will be permitted for the products in this section.

    - Substitutions: [Under provisions of Division 1.] [Not permitted.]

  - COMPONENTS

- Include the following for roofs using S-5! non-penetrating attachment clamps. Do not use S-5! Mini Clamps for the attachment of ColorGard.

    - Clamps:

      - Manufactured from 6061-T6 aluminum extrusions conforming to ASTM B221 or aluminum castings conforming to ASTM B85 and to AA Aluminum Standards and Data.

      - Clamp model: No. [S-5-U.] [S-5-T.] [S-5-Z.] [S-5-E.] [S-5-K.] [S-5-R.] [S-5-T2.]

      - Set screws: 300 Series stainless steel, 18-8 alloy, 3/8 inch diameter, with round nose point.

      - Attachment bolts: 300 Series stainless steel, 18-8 alloy, 10 mm diameter, with flat washers.

- **** OR ****

- Include the following for exposed fastener metal panels or metal tiles and shingles using a face-fastened (penetrating) attachment.

    - Brackets:

      - Manufactured from 6061-T6 alloy and temper aluminum extrusions conforming to ASTM B221 and AA Aluminum Standards and Data or cast aluminum.

      - Model: VersaBracket.

- The following screws are not provided by Metal Roof Innovations, Ltd., but must be compatible with VersaBracket.

      - Screws for attachment of brackets to roof: [Stainless steel,] [____,] of type best suited to application.

    - Cross Members:

      - Manufactured from 6061-T6 alloy and temper aluminum extrusions conforming to ASTM B221 and AA Aluminum Standards and Data.

      - Receptacle in face to receive color-matched metal strips.

      - Provide splice connectors ensuring alignment and structural continuity at end joints.

    - Color Strips: Same material and finish as roof panels; obtained from roof panel manufacturer.

- Snow and ice clips are required for standing seam roofs with seam heights of 2 inches or more and are optional for seam heights less than 2 inches.

    - Snow and Ice Clips: Aluminum, with rubber foot, minimum 3 inches wide.

- - EXECUTION

  - EXAMINATION

- Include the following for roofs using S-5! attachment.

    - Prior to beginning installation, verify that:

      - Panel seaming is complete.

      - Panel attachment is sufficient to withstand loads applied by snow guard system.

      - Installation will not impeded roof drainage.

- **** OR ****

- Include the following for roofs using VersaBracket attachment.

    - Prior to beginning installation, verify that:

      - Roof attachment is sufficient to withstand loads applied by snow guard system.

      - Installation will not impeded roof drainage.

  - PREPARATION

    - Clean areas to receive attachments; remove loose and foreign matter that could interfere with installation or performance.

- Include the following for roofs using S-5! clamp attachment.

  - INSTALLATION

    - Install system in accordance with manufacturer's instructions and approved Shop Drawings.

    - Place clamps at maximum 32 inches on center or as required by in-service loads.

    - Place clamps in straight, aligned rows.

    - Place both set screws on same side of clamp.

    - Tighten set screws to manufacturer's recommended torque. Randomly test set screw torque using calibrated torque wrench.

    - Insert color-matched metal strips into cross members, staggering strips to cover cross member joints.

    - Attach cross members to clamps; tighten bolts to manufacturer's recommended torque.

    - Install splice connectors at cross member end joints.

    - Do not cantilever cross members more than 4 inches beyond last clamp at ends.

- Include the following when applicable. Use one SnoClip per panel for panels up to and including 24 inch seam spacing and two per panel over 24 inches.

    - Install [one SnoClip] [two SnoClips] per panel between panel seams.

- **** OR ****

- Include the following for roofs using VersaBracket attachment.

  - INSTALLATION

    - Install system in accordance with manufacturer's instructions and approved Shop Drawings.

    - Place clamps at maximum 32 inches on center or as required by in-service loads.

    - Place clamps in straight, aligned rows.

    - Insert color-matched metal strips into cross members.

    - Attach cross members to brackets using self-tapping screws provided.

    - Install splice connectors at cross member end joints.

    - Do not cantilever cross members more than 4 inches beyond last VersaBracket at ends.

- Include the following when applicable.

    - Install SnoClips at spacings indicated on Shop Drawings.

- END OF SECTION